In reference to your proposed microcontroller application study, design the schematic diagram, pictorial diagram, computations, and C++ program. Simulate and take a video recording on the functional operation of the system

This simple simulation system features a set of buttons, different color LEDs, resistors, 1 buzzer and an Arduino Uno Board. The 5 buttons represents the 5 sensors that will be used in the system, namely; Temperature (DS18B20), Humidity (DHT22), Smoke (MQ-2), Gas (MQ-9), Oxygen (ZE03-O2).  These buttons will be represented by a set of LEDs and 1 buzzer. If the temperature button is pressed, a red colored LED will light and the buzzer will go high (there will be different frequency per button (sensors) just for categorization). Lastly, there will be a main button wherein if pressed, will light up all the LEDs and turn on the buzzer (which signify that all sensor sensed a change in the environment). Below is the representation of the simulation logic:

| Buttons | Simulated Sensor | LED Color | Buzzer Tone |
| --- | --- | --- | --- |
| Button 1 | Temperature (DS18B20) | Red | 1000hz |
| Button 2 | Humidity (DHT22) | Blue | 800hz |
| Button 3 | Smoke (MQ-2) | White | 1200hz |
| Button 4 | Gas (MQ-9) | Yellow | 900hz |
| Button 5 | Oxygen (ZE03-O2) | Green | 2000hz |
| Button 6 | All sensor | All LEDs | Default |

Below are the pin connection of the various components to the Arduino Uno R3:

| Component | Arduino Pin |
| --- | --- |
| Temperature Button | 2 |
| Humidity Button | 3 |
| Smoke Button | 4 |
| Gas Button | 5 |
| Oxygen Button | 6 |
| Main Button | 7 |
| Temperature LED (Red) | 8 |
| Humidity LED (Blue) | 9 |
| Smoke LED (White) | 10 |
| Gas LED (Yellow) | 11 |
| Oxygen LED (Green) | 12 |

The following information are the computations for the necessary components (resistor). Using the formula R=Vsupply-Vled/I (required current of LED). We have calculated the necessary resistor value per led:

| LED Color | VLED | I | Computed R | Nearest Value |
| --- | --- | --- | --- | --- |
| Red | 2.0V | 0.02A | (5 V – 2 V)/0.02 A = 150 Ω | 180 Ω or 220 Ω |
| Yellow | 2.1V | 0.02A | (5 – 2.1)/0.02 = 145 Ω | 180 Ω or 220 Ω |
| Green | 2.2V | 0.02A | (5 – 2.2)/0.02 = 140 Ω | 180 Ω or 220 Ω |
| Blue | 3.2V | 0.02A | (5 – 3.2)/0.02 = 90 Ω | 180 Ω or 220 Ω |
| White | 3.0V | 0.02A | (5 – 3.0)/0.02 = 100 Ω | 180 Ω or 220 Ω |

The following is the respective pictorial and schematic diagram of the aforementioned circuit:

The following is the code of the aforementioned circuit:

// Define pins

const int buttonPins[5] = {2, 3, 4, 5, 6};   // Sensor buttons

const int ledPins[5]    = {8, 9, 10, 11, 12}; // Sensor LEDs

const int mainButton = 7;                     // Main alarm button

const int buzzer = 13;                        // Single buzzer

// Frequencies for each sensor

const int frequencies[5] = {1000, 800, 1200, 900, 700};

void setup() {

for (int i = 0; i < 5; i++) {

pinMode(buttonPins[i], INPUT);

pinMode(ledPins[i], OUTPUT);

}

pinMode(mainButton, INPUT);

pinMode(buzzer, OUTPUT);

}

void loop() {

// Main alarm button pressed?

if (digitalRead(mainButton) == HIGH) {

activateAllAlarms();

} else {

bool anyActive = false;

// Check each "sensor"

for (int i = 0; i < 5; i++) {

if (digitalRead(buttonPins[i]) == HIGH) {

digitalWrite(ledPins[i], HIGH);

tone(buzzer, frequencies[i]); // Each has unique tone

anyActive = true;

delay(100); // Small delay for clarity

} else {

digitalWrite(ledPins[i], LOW);

}

}

// Turn off buzzer if no buttons are pressed

if (!anyActive) {

noTone(buzzer);

}

}

}

// Main alarm function

void activateAllAlarms() {

// Turn all LEDs ON

for (int i = 0; i < 5; i++) {

digitalWrite(ledPins[i], HIGH);

}

// Strong general alarm sound

tone(buzzer, 2000);

delay(1000);

// Turn everything OFF

for (int i = 0; i < 5; i++) {

digitalWrite(ledPins[i], LOW);

}

noTone(buzzer);

}